# OSPF Routing Configuration

Disclaimer: this was the second practical exercise done in class. This activity belongs to Eduvos – I just wanted to show the skills learnt throughout my degree.

Step 1: Add and Connect the Devices

| DEVICES: | DEFAULT NAME: | NEW NAME: |
| --- | --- | --- |
| PT-Empty Router | Router0 | R0 |
|  | Router1 | R1 |
|  | Router2 | R2 |
| 2960-24TT Switch | Switch0 | S0 |
|  | Switch1 | S1 |
|  | Switch2 | S2 |
| PC-PT PC | PC0 | PC0 |
|  | PC1 | PC1 |
|  | PC2 | PC2 |
|  | PC3 | PC3 |
|  | PC4 | PC4 |
|  | PC5 | PC5 |

Step 2: Add Ports in Routers

Steps:

- Switch off the device.

- Add the ports.

- Switch on the devices.

Router0 Before:

Router0 After:

Router1 Before:

Router1 After:

Router2 Before:

Router2 After:

Step 3: Connect the Devices

Use a serial DCE for the routers and a copper straight-through cable was used to connect the other devices.

- Router0 (Serial1/0) to Router1 (Serial1/0)

- Router1 (Serial2/0) to Router2 (Serial1/0)

- Router0 (Serial2/0) to Router2 (Serial2/0)

- Switch0 (FastEthernert0/1) to PC0 (FastEthernert0)

- Switch0 (FastEthernert0/2) to PC1 (FastEthernert0)

- Switch1 (FastEthernert0/1) to PC2 (FastEthernert0)

- Switch1 (FastEthernert0/2) to PC3 (FastEthernert0)

- Switch2 (FastEthernert0/1) to PC4 (FastEthernert0)

- Switch2 (FastEthernert0/2) to PC5 (FastEthernert0)

- Router0 (FastEthernet0/0) to Switch0 (FastEthernert0/3)

- Router1 (FastEthernet0/0) to Switch1 (FastEthernert0/3)

- Router2 (FastEthernet0/0) to Switch2 (FastEthernert0/3)

Step 4: Assign IP addresses to the PCs

PC > Desktop > IP Configuration > Enter IPv4 details

PC0:

- IPv4 Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Screenshot of Output

PC1:

- IPv4 Address: 192.168.1.3

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Screenshot of Output

PC2:

- IPv4 Address: 192.168.2.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

Screenshot of Output

PC3:

- IPv4 Address: 192.168.2.3

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

Screenshot of Output

PC4:

- IPv4 Address: 192.168.3.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.3.1

Screenshot of Output

PC5:

- IPv4 Address: 192.168.3.3

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.3.1

Screenshot of Output

Step 5: Assign IP Addresses to the cable interfaces

Step 6: Change the Interfaces

Router0 to Router1:

These configurations were done in the Router1 CLI.

Commands Used

Router> en

Router# config

Router(config)# interface serial1/0

Router(config-if)# no shutdown

Router(config-if)# end

Screenshot of Output

Router1 to Router2:

These configurations were done in the Router2 CLI.

Commands Used

Router> en

Router# config

Router(config)# interface serial1/0

Router(config-if)# no shutdown

Router(config-if)# end

Screenshot of Output

Router0 to Router2:

These configurations were done in the Router2 CLI.

Commands Used

Router> en

Router# config

Router(config)# interface serial2/0

Router(config-if)# no shutdown

Router(config-if)# end

Screenshot of Output

Router1 to Switch1:

These configurations were done in the Router1 CLI.

Commands Used

Router> en

Router# config

Router(config)# interface FastEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# end

Screenshot of Output

Router1 to Switch1:

These configurations were done in the Router2 CLI.

Commands Used

Router> en

Router# config

Router(config)# interface FastEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# end

Screenshot of Output

Step 7: Configure the Interfaces in Router0

Commands Used

R0> en

R0# config

R0(config)# interface FastEthernet0/0

R0(config-if)# ip address 192.168.1.1 255.255.255.0

R0(config-if)# no shutdown

R0(config-if)# exit

R0(config)# interfaceserial1/0

R0(config-if)# ip address 10.0.0.1 255.0.0.0

R0(config-if)# clock rate 64000

R0(config-if)# no shutdown

R0(config-if)# exit

R0(config)# interfaceserial2/0

R0(config-if)# ip address 12.0.0.1 255.0.0.0

R0(config-if)# clock rate 64000

R0(config-if)# no shutdown

R0(config-if)# exit

Screenshot of Output

Step 8: Configure the OSPF in Router0

Commands Used

Router0> en

Router0# config

Router0(config)# router ospf 1

Router0(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router0(config-router)# network 10.0.0.0 0.255.255.255 area 0

Router0(config-router)# network 12.0.0.0 0.255.255.255 area 0

Screenshot of Output

Step 9: Configure the Interfaces in Router1

Commands Used

Router> en

Router# config

Router(config)# interface FastEthernet0/0

Router(config-if)# ip address 192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface serial2/0

Router(config-if)# ip address 11.0.0.1 255.0.0.0

Router(config-if)# clock rate 64000

Router(config-if)# no shutdown

Router(config-if)# exit

Screenshot of Output

Step 10: Configure the OSPF in Router1

Commands Used

Router> en

Router# config

Router(config)# router ospf 1

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.255.255.255 area 0

Router(config-router)# network 11.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Screenshot of Output

Step 11: Configure the interfaces in Router2

Commands Used

Router> en

Router# config

Router(config)# interface FastEthernet0/0

Router(config-if)# ip address 192.168.3.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface serial2/0

Router(config-if)# ip address 12.0.0.1 255.0.0.0

Router(config-if)# clock rate 64000

Router(config-if)# no shutdown

Router(config-if)# exit

Screenshot of Output

Step 12: Configure the OSPF in Router2

Commands Used

Router> en

Router# config

Router(config)# router ospf 1

Router(config-router)# network 192.168.3.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.255.255.255 area 0

Router(config-router)# network 11.0.0.0 0.255.255.255 area 0

Router(config-router)# exit

Screenshot of Output